Alvis Stalwart, Mk 2, FV622

Detailed Activity Guide for Removing and Refitting

Of Main Engine Assembly

Introduction

- This supplement is a detailed activity guide for two tradesmen for removing and refitting of a main engine assembly. The method detailed does not supersede or cancel any previously published data.

- The method and time schedule have been developed primarily for the guidance of REME field workshops and forward repair teams. The operation was carried out in conditions normally found in a field workshop operating in a static role, by average skilled personnel of limited experience on this equipment working at standard rates. Equipments selected for studies were in a fair and reasonable condition and situated on level ground.

Table 1 – Times

| Detail | Direct Labour |
| --- | --- |
| Throughput time of task Productive man-hours – Tradesmen Net Productive man-hours - Tradesmen | 7.5 hours 15 hours 15 hours |
| Times quoted above are for cases where work is undertaken in normal conditions by average personnel working at standard rates |  |

- Where conditions vary from those previously mentioned, suitable rest and contingency allowances must be given in accordance with current practice. Skilled personnel familiar with the equipment should improve the throughput time of this guide

- Due to the non availably of a new engine assembly the operation of packing, preservation and preparation has not been included in this guide. The task of packing and preservation is normally carried out by a skilled labourer.

- The legend shown in Table 2 is used throughout the supplement to show crew locations and working arrears.

Table 2 – Legend

| Symbol | Definition |
| --- | --- |
| I O D | Inside Vehicle Outside Vehicle Drivers Compartment |

- A detailed list of stores and equipment required to carry out this task is shown in Table 3. Expense items are to be renewed as found necessary and a suitable supply of these stores should be made available.

- The importance of providing a good workplace layout for each individual job cannot be over emphasized. Time can be saved by intelligent positioning of tools, equipment and vehicle components throughout the process.

- The following abbreviations have, in some instances, been used in the activity guide.

OJ	 	- Open Jaw Spanner

DE 	 	- Double Ended Spanner

Comb. 		- Combination Spanner

LH 		- Left hand side of vehicle, facing forward 	   from driving position

RH		- Right hand side of vehicle, facing forward 	   from driving position

OP

No		- Operation number of activity

L		- Location

Table 3 – Stores, tools and equipment

| Item | Part Number or reference |
| --- | --- |
| Major assembly Engine assembly, gasoline, B81, No.1, Mk 8 13/2 Miscellaneous Oil, 5 gal Oil, as required | LV/10 2805998022025 OMD – 110 OM - 13 |
| Tools and equipment Toolkit, VM, basic Toolkit, VM, Supplement Jack, hydraulic, trolley type, 10 ton Sling, engine lifting Containers for oil and coolant Block, wood 36” x 12” x 6” Hooks, lifting radiators and fan housing for use in conjunction with sling, engine lifting Tool, holding fan pulleys Spanner, special, hull oil pipes Wrench, key hexagon 3/8” Strap, grip Spanner, ‘C’ adjustable Rope Sling | F1/FA 20221 F1/FA 20221 LV6/MT1 3481 Fig 1 A in U A in U Fig. 1 Fig 2 Fig 1 F1/5120 99 910 6068 LV3/AR 23000 As Available A in U |

Table 4 – Removing engine assembly

Time 3 hours

| L | OP No | Activity Fitter ‘A’ | Tools And Equipment | L | OP No | Activity Fitter ’B’ | Tools And Equipment |
| --- | --- | --- | --- | --- | --- | --- | --- |
| O | 1 | Select and position tools |  | O | 1 | Select and position tools |  |
| D | 2 | Turn off Master Switch and disconnect batteries | 9/16 AF comb | O | 2 | Remove coolant drain plug LH side of hull | 3/8” wrench key hexagon |
| O | 3 | Remove the seventeen bolts securing rear cover plate Assist fitter ‘B’ to lower plate and place to one side at rear of vehicle | 9/16” socket, 6” extension bar, and ratchet | O | 3 | Turn off two heater taps RH side of engine, open main engine coolant drain tap, RH side and drain coolant | Container Coolant |
|  |  |  |  | O | 4 | Place Jack with wood block under rear towing eye and as last bolt is removed from rear plate lower to ground |  |
| O | 4 | Unfasten clamps and lower sides of vehicle |  | O | 5 | Assist fitter ‘A’ and operate crane |  |
| O | 5 | Lift off ridge pole |  |  |  |  |  |
| O | 6 | Lift off tail gate, raise engine cover plates and lift off |  |  |  |  |  |
| I | 7 | Remove four bolts from rear and centre sections of decking, sling and lift off. Remove lose lifting eyes | 9/16” socket, ratchet, rope sling | I | 6 | Loosen bolts on air louver and lift off | ½” Socket and Ratchet |
|  |  |  |  | O | 7 | Operate crane, assist fitter ‘A’ |  |
| I | 8 | Slacken LH & RH fan hub lock nuts, hold fans and rotate pulley to free belts. Remove fan belts | Special tool Fig 2 ½” comb | O | 8 | Slacken and remove fan cowling mounting bolts LH & RH from rear of vehicle | 3/4” AF ring spanner |
| I | 9 | Loosen hose clips on air bleed and coolant overflow pipes, top LH of Radiator | Screwdriver | O | 9 | Disconnect LH & RH fire warning wires | 5/8” comb |
|  |  |  |  | O | 10 | Release clip securing coolant drain pipe to fan assembly LH rear of Hull | 7/16” comb |
| Table 4 – Removing engine assembly cont. |  |  |  |  |  |  |  |
| L | OP No | Activity Fitter ‘A’ | Tools And Equipment | L | OP No | Activity Fitter ’B’ | Tools And Equipment |
| I | 10 | Loosen and remove bolts LH & RH of fan mounting, lift out fan mounting | ¾” socket and ratchet, lifting bar and hooks fig 1 | O | 11 | Assist fitter ‘A’, operate crane |  |
| I | 11 | Loosen hose clips on RH radiator pipe | Screwdriver | O | 12 | Release radiator drain pipe LH bottom of radiator | 13/16” OE |
| I | 12 | Loosen and remove bolts LH & RH top of radiator | ¾” Socket and ratchet, ¾” comb | O | 13 | Loosen clips on bilge pump hose and disconnect hose | Screwdriver |
| I | 13 | Attach lifting gear and lift out radiator | Lifting gear as for fan mounting | O | 14 | Assist fitter ‘A’, operate crane |  |
| I | 14 | Release hose connection air cleaner to carburettor | Screwdriver | O | 15 | Loosen clip on hand bilge pump filter hose and disconnect | Screwdriver |
| I | 15 | Release air intake connection to compressor | 15/16” OJ | O | 16 | Disconnect electrical control cable from top of bilge pump | Pliers |
| I | 16 | Release two connections to dry air container | 3/8” OJ | O | 17 | Loosen and remove bolts securing bilge pump (ensure washers are retained) | 9/16” & 5/8” comb |
| I | 17 | Remove six nuts & bolts securing air cleaner assembly | ½” & 9/16” Comb | O | 18 | Remove four bolts from bilge pump filter and remove filter | 7/16” comb |
| I | 18 | Release two oil pipes from oil filter | 1”5/8” OJ | O | 19 | Remove water alarm ‘tell-tail’ switch from engine sump | ½” comb |
| I | 19 | Lift out air cleaner complete with oil filter carefully to prevent spillage of oil |  | O | 20 | Disconnect coolant outlet pipe | 1”1/8” comb |
| Table 4 – Removing engine assembly cont. |  |  |  |  |  |  |  |
| L | OP No | Activity Fitter ‘A’ | Tools And Equipment | L | OP No | Activity Fitter ’B’ | Tools And Equipment |
| I | 20 | Disconnect electrical connection to alternator | Strap grip | O | 21 | Loosen and remove the two front engine pedestal mounting bolts | 15/16” socket ratchet and universal joint |
| I | 21 | Disconnect temperature switch cable, release from clip and lay to side | 7/16” comb | O | 22 | Remove two bolts LH and RH side on top of engine mounting pedestal | ½” ring |
| I | 22 | Disconnect the two main engine oil pipes LH side of engine, top end of flexible pipes lay behind throttle bracket (note on MK 1 vehicle engine oil must be drained) | Special spanner Fig 1 | O | 23 | Loosen clips and free hose on main coolant pipe | Screwdriver |
| I | 23 | Disconnect hydraulic coke control unit, release spring and lay to side complete. This avoids bleeding on reassembly | ½” comb & pliers | O | 24 | Disconnect oil pipe, compressor to main engine, RH side of engine | Screwdriver |
| I | 24 | Disconnect hydraulic throttle connection | 5/8” comb | O | 25 | Remove pipe connections from sump | 13/16” comb |
| I | 25 | Remove pipe fuel supply to pump (ensure fuel is turned off in cab | 13/16” comb | I | 26 | Disconnect oil feed pipe to compressor RH side of engine | 11/16” comb |
| I | 26 | Disconnect oil pressure indicator pipe from LH front of engine | 5/8” comb | I | 27 | Remove coolant thermal bulb connection from front of cylinder head | 15/16” comb |
|  |  |  |  | I | 28 | Release hose clips securing main coolant pipe to top of engine | Screwdriver |
| I | 27 | Disconnect electrical lead from ignition filter unit | Strap grip | I | 29 | Release coolant bleed pipe LH rear of engine | ¾” comb |
| I | 28 | Remove coolant drain pipe from LH front side of cylinder block | 13/16” comb | I | 30 | Remove nuts securing coolant pipe clip to top of LH propulsion unit. Lay pipe to side | 7/16” comb |
| Table 4 – Removing engine assembly cont. |  |  |  |  |  |  |  |
| L | OP No | Activity Fitter ‘A’ | Tools And Equipment | L | OP No | Activity Fitter ’B’ | Tools And Equipment |
| I | 29 | Remove three electrical cables from starter motor (attach identification tabs) | Screwdriver | I | 31 | Remove nut securing pipe to compressor and remove pipe | 15/16” comb |
| I | 30 | Disconnect heater pipe from RH front of engine | 15/16” comb | I | 32 | Remove two clips securing exhaust down pipe RH side rear of engine, remove clips and distance pieces | Pliers |
| I | 31 | Remove clip securing coolant pipe to valve cover | 7/16” comb | I | 33 | Remove eight bolts and nuts from bell housing | 9/16” comb & ring |
| I | 32 | Remove bolts from compressor and hydraulic pump drive shaft (wire shaft to side of engine) | ½” socket, bar & two long extensions | I | 34 | Slacken hose clips and remove two heater hoses from LH front of engine and remove pipes | Screwdriver |
| I | 33 | Loosen and remove LH & RH front engine mounting bolts | ¾” socket, bar & two long extensions |  |  |  |  |
| I | 34 | Loosen gearbox holding down bolts | ¾” socket, bar & two long extensions | I | 35 | Lay bracket and pipes on front decking |  |
| I | 35 | Attach lifting gear to engine assembly | Lifting gear fig 1 | O | 36 | Prepare lifting gear and operate crane Note: Take care to clear brake hydraulic pipe (trailer unit) LH side of hull as engine is lifted |  |
| I | 36 | Take weight of assembly on crane and lever to rear to clear gearbox housing, raise and guide assembly clear of hull | Lever |  |  |  |  |
| O | 37 | Lower onto wooden blocks and remove lifting gear |  | O | 37 | Release Crane |  |